# BODY MEASUREMENT SYSTEM — V2 ARCHITECTURE DOCUMENTATION

## 1. Project Overview

### Objective

The goal of this system is to estimate human body measurements from images and classify the individual into one of three body types:

- Ectomorph

- Mesomorph

- Endomorph

The prediction is generated using a machine learning model trained on real anthropometric data collected over six months. The extracted measurements are also mapped into Ayurvedic body types:

- Ectomorph → Vata

- Mesomorph → Pitta

- Endomorph → Kapha

The system operates as a computer vision API that receives images from a mobile application and returns predicted body metrics and body type classification.

## 2. Core Constraint of the System

The machine learning model has already been trained using specific anthropometric features. These features cannot be modified without retraining the model. Therefore the system must always produce the following features:

- Gender

- Age

- ShoulderWidth

- ChestWidth

- Belly

- Waist

- Hips

- ArmLength

- ShoulderToWaist

- WaistToKnee

- LegLength

- TotalHeight

- BodyFatPercentage

Because of this constraint: The feature extraction pipeline must improve accuracy without changing the feature definitions.

## 3. Problems in the Previous System (V1)

The original implementation had several architectural limitations.

### 3.1 Reference Object Dependency

The system relied on detecting an A4 sheet to calculate scale.

Problems:

- A4 detection is unreliable

- Paper orientation causes distortion

- Lighting affects contour detection

- Paper may not be visible

- Users may forget to place paper
This created frequent failures in measurement extraction.

### 3.2 Single Image Measurement

The system used only one image to estimate body measurements. This introduced several geometric issues:

- No depth information

- Perspective distortion

- Incorrect width estimation

- Incorrect waist estimation
Human bodies are 3D objects, while the system attempted to measure them using 2D projections.

### 3.3 Sparse Keypoint Measurement

Measurements were derived only from pose keypoints.

Example:

- Waist = distance between hip keypoints

- Chest = approximated using shoulder width
Keypoints represent only a few anatomical points, not the full body contour. Therefore many measurements were approximations.

### 3.4 Measurement Noise

Single-frame measurement caused instability. Small detection errors produced large measurement changes.

## 4. Design Goals for the New System (V2)

The new system was designed to solve these problems while respecting the constraints of the trained ML model.

Primary goals:

- Remove dependency on reference objects

- Improve measurement accuracy

- Use geometric information from multiple views

- Stabilize measurements

- Maintain the same feature set

## 5. New System Architecture

The new pipeline introduces controlled capture and improved geometry extraction.

- Mobile Capture System

- Image Validation

- Dual Image Acquisition (Front + Side)

- Human Segmentation

- Pose Estimation

- Geometric Measurement Engine

- Feature Construction

- Machine Learning Prediction

## 6. Controlled Capture Protocol

Because the mobile application controls image capture, the system enforces strict capture conditions. These constraints significantly reduce measurement errors.

### 6.1 Camera Position

The camera must follow these rules:

- distance from subject: 2.5 meters

- camera height: waist level

- camera orientation: vertical

- camera tilt: 0°
Maintaining a fixed capture geometry allows consistent pixel-to-real-world scaling.

### 6.2 User Pose

Users must stand in a standardized posture:

- body facing forward

- arms slightly away from body (~15 degrees)

- feet shoulder width apart

- body upright
This prevents arms or clothing from blocking torso boundaries.

### 6.3 Clothing Requirements

Users must wear tight clothing.

- Recommended: gym wear, fitted shirts, sportswear.

- Prohibited: jackets, hoodies, baggy clothing, long coats.
Loose clothing distorts body measurements.

### 6.4 Lighting Requirements

Lighting must follow these conditions:

- uniform front lighting

- no strong shadows

- no backlighting
Proper lighting improves segmentation and pose detection.

### 6.5 Background Requirements

Users must stand in front of a simple background.

- Recommended: plain wall, single color background.

- Avoid: cluttered environments, mirrors, objects overlapping the body.

## 7. Dual Image Capture

Instead of using a single image, the new system captures two images:

- Front view

- Side view

Benefits:

- Front image → body width measurements

- Side image → body depth estimation
This allows the system to approximate 3D body shape.

## 8. Human Segmentation

The new system introduces a body segmentation stage.

- Recommended models: MediaPipe Selfie Segmentation, DeepLabV3.
Segmentation produces a binary silhouette mask representing the body shape.
Advantages:

- precise torso boundaries

- accurate waist width detection

- improved chest width detection
This replaces many approximations from the previous system.

## 9. Pose Estimation

Pose estimation continues to use YOLOv8 Pose. Keypoints extracted:

- shoulders, hips, knees, ankles, elbows, wrists.
These landmarks are used for length measurements.

## 10. Measurement Extraction

Measurements are computed using a combination of pose landmarks and segmentation boundaries.

- Shoulder Width: distance between left shoulder and right shoulder (Adjusted using silhouette boundaries).

- Waist Width: Computed from the narrowest point of the torso in the segmentation mask.

- Chest Width: Measured from the silhouette at chest level.

- Arm Length: Computed as: shoulder → elbow → wrist

- Leg Length: Computed as: hip → knee → ankle

- Vertical Measurements: Computed using midpoint distances (e.g., ShoulderToWaist, WaistToKnee).

## 11. Depth Estimation Using Side Image

Side images allow estimation of body depth (e.g., waist_depth, chest_depth, hip_depth). Circumference can then be approximated using an ellipse model.

Ellipse circumference approximation:

$$C \approx \pi \cdot (3(a + b) - \sqrt{(3a + b)(a + 3b)})$$

Where:

- a = front width / 2

- b = side width / 2
This provides better approximation of real body circumference.

## 12. Height Normalization

Since the system no longer uses a reference object, height is used as a scaling factor. Users manually input their height.

Scaling factor:

$$scale = \frac{\text{real\_height}}{\text{pixel\_height}}$$

All body measurements are scaled using this factor.

## 13. Measurement Stabilization

To reduce noise, multiple frames are captured.

Procedure:

- capture 5 frames

- compute median measurement
Median filtering removes pose detection noise.

## 14. Feature Construction

Once measurements are computed, the required features are assembled:

- Gender, Age, ShoulderWidth, ChestWidth, Belly, Waist, Hips, ArmLength, ShoulderToWaist, WaistToKnee, LegLength, TotalHeight, BodyFatPercentage.
The feature format remains unchanged to maintain compatibility with the trained ML model.

## 15. Machine Learning Prediction

The system loads the trained model:

- bodytype_model.pkl

- label_encoder.pkl

Prediction output: Ectomorph, Mesomorph, or Endomorph. The predicted label is then mapped to Ayurvedic types.

## 16. Components Removed from Previous System

- A4 reference object detection

- Contour-based scale estimation

- Single-image measurement pipeline

- Several heuristic width approximations

## 17. Components Added in the New System

- Controlled capture protocol

- Dual image capture

- Human segmentation

- Depth estimation using side view

- Multi-frame measurement stabilization

- Image validation checks

## 18. Expected Performance Improvements

Estimated measurement error reduction:

| System Version | Error Rate |
| --- | --- |
| Previous system | 20–35% |
| Controlled capture | 10–15% |
| Dual-view measurement | 5–10% |

Accuracy improvements result from better geometry extraction.

## 19. System Limitations

- Clothing: Loose clothing can still distort body boundaries.

- User Compliance: Accuracy depends on users following capture instructions.

- Camera Hardware Variability: Different phone cameras may introduce slight measurement inconsistencies.

- Depth Approximation: Side-view estimation approximates depth but does not fully reconstruct 3D body geometry.

## 20. Future Improvements

Potential upgrades include:

- 3D body reconstruction (SMPL models)

- depth estimation using neural networks

- automatic camera calibration

- real-time capture validation

## 21. Summary

The new system improves measurement accuracy by removing fragile reference object detection, introducing controlled capture conditions, using two-view geometry, adding body segmentation, and stabilizing measurements. The machine learning model remains unchanged while the feature extraction pipeline becomes significantly more reliable.